| Design, implementation and reference TusmuertOS 2.0 | Abstract Technical overview of the design and implementation of a hobbyist OS. This is a learning project. mihe TusmuertOS 2.0 |
| --- | --- |

Contents

2	TusmuertOS 2.0 design	3

3	Bootloader	3

4	Helpers and auxiliary tools	4

4.1	FAT32 image creator	4

5	Misc and notes	8

5.1	Random notes	8

5.1.1	BOCHS	8

5.1.2	STORAGE	10

5.2	WSL stuff	14

5.3	Setup build	14

5.4	BUILD PROCESS BEFORE MAKE	17

5.5	BUILD PROCESS AFTER MAKE	17

5.6	CPU registers	17

6	Common Linux and Windows commands	18

6.1	Linux	18

6.2	Windows PowerShell	19

7	Random nuggets of knowledge	20

7.1	How Org directive works in NASM assembly	20

7.2	WSL Tab autocomplete searchs only on windows/system32	20

7.3	On bootloader, will mov sp, 0x7C00 make the next push to overwrite the first byte of the bootloader? (Overlap)	20

7.4	Full PowerShell history	21

7.5	Git inconsistency between files on the same folder via Windows and WSL on VSCODE	21

7.6	Hint C debugger that a buffer is of a particular size	21

7.7	Frame-Pointer Omission	21

7.8	How to create a hard disk image file with FAT32. Interesting boot sequence on MBR	21

7.9	Return to the last working directory. Windows and linux	25

7.10	Forcing a pointer to be interpreted as a type does have a name: Type punning	25

8	Research backlog	25

9	Screenshots	26

10	Resources	32

11	Clocked-in hours	33

12	Table of figures	33

# TusmuertOS 2.0 design

- Target architecture: x86_64 (AMD64)

- Execution mode: Real mode (16 bits) >>> Protected mode (32 Bits) >>> Long mode (64 bits)

- Endianness: Little-endian

- Word size: 64bits

- Boot platform: BIOS or UEFI. Probably BIOS for Bochs compatibility.

- Bootloader: Custom bootloader

- Kernel type: Monolithic

- Build system: make or modern alternative

- Language: c for simplicity, Rust/Zig for challenge. NASM assembly for bootloader.

- Assembly syntax: Intel syntax

- Toolchain: GCC cross-compiler or rust freestanding. NASM for assembly.

- Target-triplet: x86_64-unknown-tusmuertos

- Target platform: BOCHS >> QEMU >> HYPER-V >> rest of hypervisors

- Calling convention on bootloader

- Calling convention on Kernel

- Other tools:

  - Hex editor: HIMEX

  - Text editor: Notepad++, Visual Studio Code

  - IDE Assembly: Visual Studio Code

  - IDE C or Rust: Visual Studio Code or Visual Studio 2022

  - Image, bin editor, concatenation, elf info, debug, tracer, etc……

# Bootloader

- General:

  - Language: NASM Assembly

  - Toolchain: NASM (Linux or windows, preferably Linux on WSL)

  - 2 Stages. Minimal first stage until Protected mode + Second stage to Long mode

    - Stage1: 16 bits to 32 bits Real to Protected. NASM on 16 bits, only code on 32 bits is the jump to the second stage.

- Functionality:

  - Stage1:

    - BIOS loads at 0x7C00 (0x07C0 x 0x0000) ✅

    - Verify 0x55AA signature (0101 0101 1010 1010) ✅

    - CPU registers and flags initialization ✅

    - Set a temporary stack. Probably 0x7C00 downwards. ✅

    - Print early status ✅

    - Detect storage drives. Probably only one at the beginning.

    - Storage fixed LBA or simple Fat12/26/32????

    - Jump to Protected mode. Maybe change this to the second stage?

    - Load stage 2. Target memory TBD

    - Jump to Stage 2

  - Stage2:

    - Jump to Protected mode if Stage1 did not change mode.

    - Enable A20, or at least check. There are multiple strategies for this. Some platforms have fast enable 0x92, or they come with A20 already enable. XXXX

    - Create console helpers for printing and stuff.

    - Gather info for the kernel: Memory, graphics, bios, acpi, others…

    - Load kernel:

      - Flat binary: Explore this option, maybe is simpler

      - ELF: Parser + segments + etc…

    - Define kernel entry

    - Set the GDT and Jump to protected mode

    - Verify if long mode if possible, for completeness

    - Enable paging

    - Enable PAE

    - Jump to Long mode

  - Optional, but style points:

    - Error handling

    - Checksums

# Helpers and auxiliary tools

## FAT32 image creator

Revamp this section to multiple small tools:

- Empty disk creator

- FAT32 formater (with the idea to include FAT12/16, HD/Floppy)

- Binary injector on offset

- One file adder

Purpose and scope

- Automate the process of creating a binary image file to load as floppy/usb.

- It creates an MBR and VBR(with BPB) to boot and have a FAT12/16/32 structure

- Once the FAT structure is ready, copy (inject) some files onto the file system.

Technical details

- Language: C or Rust

- Runtime: Linux (explore Windows compatibility)

- Binary Name: fatimagecreator

- CLI

- If Rust:

  - Crates:

    - Clap, to handle the command line arguments

- Process:

  - Take the list of arguments and split it on: sector0.bit files_to_add, output file, floppy or hd, sector0.bin

  - Create a file in the same folder invoking dd with the requested size and zeroed out.

  - For floppy use FAT12, for HD use FAT32

  - Forge valid FAT descriptors based on format 12/32

  - Inject the FAT structure on the disk

  - Inject the sector0.bin on offset 0x00.

  - Add the list of files.

Operation

- Use a list of files as arguments and the last is the output file.

- Fatimagecreator file01 file02 file03 file03 outputfile.img floppy/hd size_mb(1.44MB/xxxMB) bootloader.bin

Internals

FAT32 structure details:

- MBR:

| General Offset | Relative Offset | Section | Bytes | Total Bytes | Comments | Value |
| --- | --- | --- | --- | --- | --- | --- |
| 0x00000000 | 0x0000 | MBR | 446 | 446 | Booting code | xxxxxxxxxxxxxxxxxx |
| 0x000001BE | 0x01BE | MBR | 16 | 462 | 1st Partition entry |  |
| 0x000001CE | 0x01CE | MBR | 16 | 478 | 2nd Partition entry |  |
| 0x000001DE | 0x01DE | MBR | 16 | 494 | 3rd Partition entry |  |
| 0x000001EE | 0x01EE | MBR | 16 | 510 | 4th Partition entry |  |
| 0x000001FE | 0x01FE | MBR | 2 | 512 | Boot magic 0xAA55(0x55 0xAA Little-e) |  |

- 1st Partition entry:

| General Offset | Relative Offset | Section | Bytes | Total Bytes | Comments | Value |
| --- | --- | --- | --- | --- | --- | --- |
| 0x000001BE | 0x0000 | 1st Partition entry | 1 | 1 | State of partition (080=Active/Boot) | xxxxxxxxxxxxxxxxxx |
| 0x000001BF | 0x0001 | 1st Partition entry | 1 | 2 | Beginning partition HEAD |  |
| 0x000001C0 | 0x0002 | 1st Partition entry | 2 | 4 | Beginning partition CYLINDER/SECTOR |  |
| 0x000001C2 | 0x0004 | 1st Partition entry | 1 | 5 | Partition type | 0x0B |
| 0x000001C3 | 0x0005 | 1st Partition entry | 1 | 6 | End partition HEAD |  |
| 0x000001C4 | 0x0006 | 1st Partition entry | 2 | 8 | End partition CYLINDER/SECTOR |  |
| 0x000001C6 | 0x0008 | 1st Partition entry | 4 | 12 | Gap in sectors from MBR to First sector in the partition |  |
| 0x000001CA | 0x000C | 1st Partition entry | 4 | 16 | Number of sectors in partition |  |

- Boot record (VBR)

| General Offset | Relative Offset | Section | Bytes | Total Bytes | Comments | Value |
| --- | --- | --- | --- | --- | --- | --- |
| 0x00000000 | 0x0000 | VBR |  |  | Short jump + NOP | xxxxxxxxxxxxxxxxxx |
|  |  | VBR |  |  |  |  |
|  |  | VBR |  |  |  |  |
|  |  | VBR |  |  |  |  |
|  |  | VBR |  |  |  |  |
|  |  | VBR |  |  |  |  |

Figure 1 Here is a canonical example of a VBR from Microsoft.

# Misc and notes

## Random notes

### BOCHS

- Bochs require a config file to configure the hardware. That config file also points to the image to load.

- -q to avoid the hardware configuration screen on load (config comes from the file)

- -q debugger (you must add it also to the config file)

- Config file example. It may require some changes, but it works.:

display_library: win32, options="traphotkeys, autoscale, gui_debug"

cpu: model=core2_penryn_t9600, count=1, ips=50000000, reset_on_triple_fault=1, ignore_bad_msrs=1, msrs="msrs.def"

cpu: cpuid_limit_winnt=0

memory: guest=512, host=256, block_size=512

romimage: file=$BXSHARE/BIOS-bochs-latest, options=fastboot

vgaromimage: file=$BXSHARE/VGABIOS-lgpl-latest.bin

keyboard: type=mf, serial_delay=150

mouse: enabled=0

pci: enabled=1, chipset=i440fx

private_colormap: enabled=0

floppya: 1_44=C:\Users\mihe\Dropbox\TusMuertOS-Oct2025\src\build\floppy.img, status=inserted, write_protected=1

boot: floppy

floppy_bootsig_check: disabled=0

log: C:\Users\mihe\Dropbox\TusMuertOS-Oct2025\src\build\bochsout.txt

panic: action=ask

error: action=report

info: action=report

debug: action=ignore, pci=report # report BX_DEBUG from module 'pci'

debugger_log: -

parport1: enabled=1, file="parport.out"

sound: driver=default, waveout=/dev/dsp. wavein=, midiout=

speaker: enabled=1, mode=sound, volume=15

magic_break: enabled=1

megs: 16

### STORAGE

- Non-partitioned storage, such as floppy, use VBR. First sector, physical C/H/S 0,0,1 . Note the sector counting begins with 1 not 0.

- Exercise done:

  - Formated a USB drive:

    - MSDOS boot

    - BIOS

    - MBR partition scheme

    - FAT32

    - 16kb Cluster size

  - Open the Disk using HxD (Open disk option is brilliant!)

  - Sector 0 contains the MBR, sector 2048 contains the BPB of a partition. Both can be see easily using HxD

  - Partition information extracted from the 4x16 bytes before the boot magic number. The first value begins at 0x1BE.

  - Exported first 512 to a file using HxD, then used ImHEx disassembler feature to see the code. 16bits-x86-LitteEndian.

Figure 2 Example of partition on MBR. 0x80 is bootable

Figure 3 Dissection of the first partition using ChatGPT. Is correct?

Figure 4 MBR with FAT partitions

Figure 5 FAT formats schema

- This is how I interpret the disk structure based on my tests:

Disk FAT32 - MSDOS BOOT - RUFUS:

Sector 0    MBR

- Boot code

- Partition table (starts 0x1BE)

Sector 2048 Volume Boot Record (VBR)

- Jump + OEM

- BPB (filesystem metadata)

- Possibly bootstrap code

Sector 2049 FAT, Root Dir, Data ...

## WSL stuff

- File system:

  - Accessing Windows from Linux:

    - Console:

      - Invoke explorer Windows using “explorer.exe .”

      - Access Windows drives through the mounted drive: /mnt/c/Program Files (x86)

    - Running directly from Linux console:

      - Example: “ipconfig.exe | grep IPv4 | cut -d: -f2”

  - Accessing Linux from Windows:

    - Console

      - Using PowerShell: cd \\wsl.localhost\Ubuntu\home

      - Also you can net use Z: \\wsl.localhost\Ubuntu

    - Explorer

    - Running directly from windows console:

      - Example: “wsl ls -la” will output Windos folder but using Linux ls

      - Example: wsl ls -la | findstr "git"

## Setup build

- Install IMHEX hex editor. https://imhex.werwolv.net/

- Install WSL, Ubuntu, v2.

- Update WSL:

  - sudo apt update

  - sudo apt upgrade -y

  - sudo apt full-upgrade -y

  - sudo apt autoremove -y

- Install NASM assembler:

  - sudo apt install nasm -y

- Install bximage:

  - sudo apt install bximage

- Install make:

  - sudo apt install make

- Create a temporary alias for cls = clear(investigate how to make it permanent)

  - alias cls='clear'

- Install qemu on Windows: https://www.qemu.org/ https://qemu.weilnetz.de/w64/

  - Current version, October 2025 is 10.1.2

- Install Bochs, version 3.0 October 2025 https://github.com/bochs-emu/Bochs/releases

  - Installation path: C:\Program Files\Bochs-3.0

- Install gcc on WSL:

  - Sudo apt install gcc

- Install build-essential and gdb:

  - sudo apt-get install build-essential gdb

- Enable GUI for WSL. (Using remote WSL/VSCODE is insufferable):

  - READ ALL STEPS BEFORE STARTING THE FIRST

  - wsl –update

  - wsl –shutdown

  - sudo snap install --classic code # or code-insiders

  - type -a code

    - code is /mnt/c/Program Files/Microsoft VS Code/bin/code

    - code is /snap/bin/code

  - sudo mv /usr/bin/code /usr/bin/code-windows

  - (if I want to restore the command above) sudo mv /usr/bin/code-windows /usr/bin/code

  - Actually, what worked was [interop] appendWindowsPath=false

  - Wsl –shutdown again.

  - After these steps I lost the ability to run windows commands from WSL console, such as IPCONFIG.

  - Now using code . will open native, with a warning.

  - sudo apt install nautilus (because why not)

  - sudo apt install -y firefox (to complete the oauth github auth)

- Prepare vscode on Linux for c:

  - Create a launch and tasks jsons.

  - Watch if defender is blocking the creation of binaries. Exclude the folder if required.

  - {

  - // Use IntelliSense to learn about possible attributes.

  - // Hover to view descriptions of existing attributes.

  - // For more information, visit: https://go.microsoft.com/fwlink/?linkid=830387

  - "version": "0.2.0",

  - "configurations": [

  - {

  - "preLaunchTask": "C/C++: gcc build active file",

  - "name": "(gdb) Launch",

  - "type": "cppdbg",

  - "request": "launch",

  - "program": "${fileDirname}/${fileBasenameNoExtension}",

  - "args": [

  - "aaaaa",

  - "bbbbb",

  - "ccccc"

  - ],

  - "stopAtEntry": false,

  - "cwd": "${fileDirname}",

  - "environment": [],

  - "externalConsole": false,

  - "MIMode": "gdb",

  - "setupCommands": [

  - {

  - "description": "Enable pretty-printing for gdb",

  - "text": "-enable-pretty-printing",

  - "ignoreFailures": true

  - },

  - {

  - "description": "Set Disassembly Flavor to Intel",

  - "text": "-gdb-set disassembly-flavor intel",

  - "ignoreFailures": true

  - }

  - ]

  - }

  - ]

  - }

  - {

  - "tasks": [

  - {

  - "type": "cppbuild",

  - "label": "C/C++: gcc build active file",

  - "command": "/usr/bin/gcc",

  - "args": [

  - "-fdiagnostics-color=always",

  - "-g",

  - "${file}",

  - "-o",

  - "${fileDirname}/${fileBasenameNoExtension}"

  - ],

  - "options": {

  - "cwd": "${fileDirname}"

  - },

  - "problemMatcher": [

  - "$gcc"

  - ],

  - "group": {

  - "kind": "build",

  - "isDefault": true

  - },

  - "detail": "Task generated by Debugger."

  - }

  - ],

  - "version": "2.0.0"

  - }

- Install Mtools for FAT image management:

  - Sudo apt install mtools

- Install rust toolchain:

  - sudo curl --proto '=https' --tlsv1.2 -sSf https://sh.rustup.rs | sh

## BUILD PROCESS BEFORE MAKE

- Compile the bootloader stage1 that goes on MBR to mbr.bin

- Compile the bootloader stage2 that goes on VBR to vbr.bin

- Compile the Kernel to kernel.bin

- Create an empty image file, 8MB, Zeroed

- Format it as FAT32 with 1 partition of 6MB, including MBR.

- Inject mbr.bin on the disk MBR at sector 0

- Inject vbr.bin on the disk at sector after the MBR + reserved gap.

- Add, as file on the file system FAT32 the file kernel.bin directly on the disk root

## BUILD PROCESS AFTER MAKE

## CPU registers

All registers

The registers I care about

## Rust notes

How to handle cases:

- Need to handle multiple cases? → use match

- Need to handle one specific pattern? → use if let

- Need to check a simple condition? → use if

- Need to repeat until condition false? → use while

- Need to repeat while pattern matches (like pop from stack)? → use while let

- Need infinite or manual loop control? → use loop

- Iterating over a collection? → use for

Change color of “?” (Control flow for Result and Option):

"editor.semanticHighlighting.enabled": true,

"editor.semanticTokenColorCustomizations": {

"rules": {

"operator.controlFlow": {

"foreground": "#ff5100",

"bold": true

}

}

},

# Common Linux and Windows commands

<TODO> Convert this to table format

## Linux

Build binary from asm file using NASM. Relative path, the full path for this is:

/mnt/c/Users/mihe/Dropbox/TusMuertOS-Oct2025

nasm -f bin src/bootloader/stage1.asm -o src/build/stage1.bin

bximage to create an empty floppy disk. It is an interactive tool with a menu. It shows this info after execution: The following line should appear in your bochsrc:

floppya: image="floppy.img", status=inserted

Alternative to create the empty disk:

dd if=/dev/zero of=teaching.img bs=512 count=2880

Inject the bootloader (512 bytes, One sector if I am not mistaketn) at the beginning of the empty disk image:

dd if=src/build/stage1.bin of=src/build/floppy.img bs=512 count=1 conv=notrunc

To quickly see the content of the disk:

xxd -l 512 floppy.img

## Windows PowerShell

Run qemu with the bootloader bin

qemu-system-i386 -drive format=raw,file=C:\Users\mihe\Dropbox\TusMuertOS-Oct2025\src\build\stage1.bin,if=floppy

Prepare and run Bochs

Create a Bochs configuration file, copying the template: bochsrc-sample.txt

Uncomment the lines you require with the settings you require.

Copy the template where the img is (in the file you also use full path or relative to the .img and output logs).

Run this: bochs -f .\bochsrc-runtime.txt -q -debugger

-f: configfile

-q: don’t show the configuration screen before start

-debugger: memory, registers, etc…

bochs -f .\bochsrc-runtime.txt -q -debugger

NEXT DAY !!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

bochs -f .\bochsrc-runtime.txt -q -debugger -rc .\bochs.cmds try to enable automatic breakpoint, without magic breakpoints, which take space from the bootloader.

# Random nuggets of knowledge

## How Org directive works in NASM assembly

Example:

[org 0x7C00]

bits 16

start:

mov ax, start      ; imm16 encodes the absolute offset of “start”

- With org 0x7C00, this assembles as: B8 00 7C (AX ← 0x7C00)

- With org 0x0000, it assembles as: B8 00 00 (AX ← 0x0000)

## WSL Tab autocomplete searchs only on windows/system32

No solution found yet. Tried to add to etc/wsl.conf this appendWindowsPath=false

## On bootloader, will mov sp, 0x7C00 make the next push to overwrite the first byte of the bootloader? (Overlap)

Tested it and it is safe. It will not overwrite the bootloader.

## Full PowerShell history

It is saved on the file ConsoleHost_history.txt

## Git inconsistency between files on the same folder via Windows and WSL on VSCODE

It is related to the LF vs CRLF line-endings differences.

The easiest way to resolve this would be to add .gitattributes file with a * text=auto line. This should normalize the line endings and prevent this issue within this specific repository. See this guide. Let me know if you have any other questions

## Hint C debugger that a buffer is of a particular size

buf is a pointer to uint8_t, meaning it points to a sequence of bytes (each an unsigned 8-bit value); to inspect the entire 512-byte region it references, add buf,512 to the Watch panel in VS Code so the debugger shows it as an array of bytes instead of a single uint8_t.

## Frame-Pointer Omission

Allows the compiler to use RBP (Stack Base Pointer) as general register for “optimization”, so less offset calculations and freeing other general registers on long functions. On gcc: -fno-omit-frame-pointer

Example of a prologue where a parameter coming on RDI is just saved on RBP. It may help performance, but it is difficult to reason about while reading the disassembled code.

## How to create a hard disk image file with FAT32. Interesting boot sequence on MBR

- dd if=/dev/zero of=hd16mbfat32.img bs=1M count=16

  - Zeroed, file name hd16mbfat32.img, size of chunk to write at a time, number of chunks.

- sudo parted hd16mbfat32.img mklabel msdos

  - There are only a valid list of mklabel. https://www.gnu.org/software/parted/manual/parted.html#mklabel

  - This created an interesting bootloader that relocates itself. Can be disassembled using ImHex.

  - Before continuing, only with the mklabel msdos the MBR has few strange characters before the partitions table. Some kind of signature… 4 bytes, little Indian. This is a Microsoft thing BTW.

  - sudo parted hd16mbfat32.img --script mkpart primary fat32 1MiB 100%

    - mkpart = make partition

    - primary/extended/logical

    - fat32 (there is a list on the documentation FAT16 is there, but not 12 because it is for floppies)

    - start = 1MiB end 100% (this is not very well documented)

  - parted disk16mb.img unit B print (Note there is not actual file system. Partition is there but no file system)

  - mformat -i hd16mbfat32.img@@1048576 -F ::

    - image file

    - @@Start (from the parted output)

    - -F = FAT32

    - :: Root directory of the image

  - If you open it with ImHex or Exterro FTK Imager or HxD, the FAT partition begins at 0x00100000

  - 52 52 61 41 RRaR is the FSInfo

  - This can copy files and create directories, but I have not tried. I prefer to mount it and copy normally:

    - mcopy -i disk16mb.img@@1048576 myfile.txt ::

    - mmd -i disk16mb.img@@1048576 ::/boot

    - mcopy -i disk16mb.img@@1048576 kernel.bin ::/boot

  - mdir -i hd16mbfat32.img@@1048576 ::

  - sudo losetup --offset 1048576 --find --show hd16mbfat32.img

  - losetup

  - losetup --all

  - To mount it safely without breaking /mnt:

    - sudo mkdir -p /tmp/diskimg

    - sudo losetup --offset 1048576 --find --show hd16mbfat32.img

    - ls /tmp/diskimg/

  - Explore the idea of permanent mounting via /etc/fstab

## Return to the last working directory. Windows and linux

- Linux:

  - “cd –“

- Windows:

  - Pushd or push-location

  - Move

  - Popd or pop-location

## Forcing a pointer to be interpreted as a type does have a name: Type punning

- Seriously…?

# Research backlog

- Disk formats FAT12/16/32

- Partition table on the MBR. I remember seeing adding 4 16bytes reserved structures before 0xAA55

  - I guess this applies if the MBR is for a hard disk, but does not apply for a floppy. This is an educated guess

  - On floppy, the FAT info at 0 starts with a jump to code, followed by the FAT info. This way both, the bootloader and FAT info can coexist, because there is a jump. Validate this because I got this explained by ChatGPT. It makes a lot of sense, but still…

  - A Disk with multiple partitions, the first byte define if it is bootable 0x80 = bootable. The first byte on one of those 4 16 bytes partitions at the end of the MBR before the 0xAA55

- Can MBR and BPB be fused together? I found something related to a short jump at the beginning, of BPB ??? Dive deeper into this! I think BPB merged on MBR is for disk without partitions, such as floppy disks.

- Inject breakpoints on Bochs without magic breakpoints, via commands file on startup

- How to change Bochs display resolution.

- Change the path order (open first code from linux instead of windows, without removing the windows path via etc/wsl.conf file )

- Tools for disks, the old MSDOS debug, and diskedit. AccessData FTK Imager, now with a new name Exterro FTK imager.

# Screenshots

31-October-2025

Figure 6 Quick and dirt bootloader with ABCD printed

Figure 7 Early makefile for the bootloader

01-November-2025

Figure 8 Revived my old kernel with Protected mode and keyboard support via hardware interrupts. Goose bumps…

02-November-2025

Figure 9 First steps FAT32 imaging tool

03-November-2025

Figure 10 Forged FAT32 HD Image file using Linux tooling

Figure 11 Good progress on my custom FAT32 imaging tool

04-November-2025

Figure 12 In Rust we trust (but not on this unsafe one)

05-November2025

Figure 13 Rustlings to dust off my rust skills

# Resources

| Resource | Notes |
| --- | --- |
| https://github.com/notsomeidiot123/KimisOS | bootloader made in assembly. diskwrite.c, it looks like a custom FAT32 image writer, which is an idea I am toying with. |
| https://archive.org/details/FAT_Specification/mode/1up | FAT Reference from Microsoft. Internet archive |
| https://euroassembler.eu/ | Yet another assembler. There is a ton of work behind. The macros and projects section contain a lot of pre-canned structures and asm for OsDev |
| https://elm-chan.org/docs/fat_e.html | FAT stuff |
| https://github.com/lukearend/x86-bootloader | Bootloader assembly, very basic |
| https://en.wikipedia.org/wiki/Master_boot_record | The sector layout section is a good summary. |
| docs\Reference\FileSystem\FAT-Specification-Microsoft-Aug2005.pdf | PDF FAT reference |
| https://www.easeus.com/resource/fat32-disk-structure.html?srsltid=AfmBOorykOEYj97O-GVlZWgVod0F3h0Jx9gLmAidlQA_ZXAyrPXHGyzE | Clear and concise FAT32 structures |
| https://oort.rs | Dust off your rust |
| https://github.com/usagi/rust-memory-container-cs/blob/master/3840x2160/rust-memory-container-cs-3840x2160-dark-back.png | Rosetta stone to Rust memory |

# Clocked-in hours

Hours clocked-in includes, coding, research, testing, reading, documenting, etc…

| Day | Hours aprox. |
| --- | --- |
| 31-October-2025 | 4 hours |
| 01-November-2025 | 12 hours |
| 02-November-2025 | 8 hours |
| 03-November-2025 | 6 hours |
| 04-November-2025 | 4 hours |
| 05-November-2025 | 10 hours |

# Table of figures

Figure 1 Here is a canonical example of a VBR from Microsoft.	7

Figure 2 Example of partition on MBR. 0x80 is bootable	11

Figure 3 Dissection of the first partition using ChatGPT. Is correct?	12

Figure 4 MBR with FAT partitions	13

Figure 5 FAT formats schema	13

Figure 6 Quick and dirt bootloader with ABCD printed	27

Figure 7 Early makefile for the bootloader	28

Figure 8 Revived my old kernel with Protected mode and keyboard support via hardware interrupts. Goose bumps…	28

Figure 9 First steps FAT32 imaging tool	29

Figure 10 Forged FAT32 HD Image file using Linux tooling	30

Figure 11 Good progress on my custom FAT32 imaging tool	30

Figure 12 In Rust we trust (but not on this unsafe one)	31

“Don’t use up all your resources on the easy stuff”